What are the equations for calculating plate thickness for a guitar top and back and for measure to of a guitar brace according to Trevor gore?

7:07 PM

Searched the web

Searched the web

Good — I have enough to give you a solid overview. Here's a breakdown of Trevor Gore's equations from Contemporary Acoustic Guitar Design and Build (with co-author Gerard Gilet):

## Trevor Gore's Plate Thicknessing Method

The core idea is to measure the actual material properties of the specific piece of wood you'll use (via tap tones), then calculate the target thickness that will produce consistent vibrational behavior regardless of which piece of wood you're working with.

### Step 1: Measure the Elastic Constants via Tap Tones

On a rectangular test plate (the actual piece you're going to use), you tap to find three resonant frequencies while holding the plate at its nodal points:

- f_L — longitudinal (long grain) bending mode

- f_C — cross grain bending mode

- f_LC — torsional (twist) mode

You also measure: length l, width w, thickness h, and mass (to get density ρ = mass / (l × w × h))

### Step 2: Calculate Elastic Moduli

Eq. 4.5-2 — Longitudinal Young's modulus E_L:

EL=2fLl2)2ρh2⋅π2/3

More precisely, it inverts the formula for the fundamental bending frequency of a free plate. The simplified form commonly used:

EL=48⋅ρ⋅fL2⋅l4h2⋅π2

(constants vary slightly based on boundary conditions — Gore uses specific coefficients from plate vibration theory)

Eq. 4.5-3 — Cross-grain Young's modulus E_C: Same form, substituting f_C and w (width) for the cross-grain direction.

Eq. 4.5-4 — Shear modulus G_LC:

GLC=1.21585⋅ρ⋅l2⋅w2⋅fLC2h2

### Step 3: Calculate Target Plate Thickness

Eq. 4.5-7 — Target thickness h (the key result):

h=0.95977⋅f⋅a2⋅ρEL+ab4EC+ab2(0.02857⋅EL+1.12⋅GLC)

Where:

- f = "vibrational stiffness" parameter — a tuning number you choose (Gore uses ~75 for steel-string tops, ~180,000-ish for classical; units need to be consistent)

- a = body length of the guitar (neck join to tail, in meters)

- b = body width (in meters)

- ρ = density of the wood

- E_L, E_C, G_LC = the moduli calculated above

Important: The a and b dimensions here are the finished guitar body dimensions, NOT the test plate dimensions. The test plate dimensions are only used in 4.5-2 through 4.5-4.

## Brace Depth / Second Moment of Area

For braces, Gore uses structural beam mechanics. The key concepts are in section 4.4:

Flexural rigidity (bending stiffness) of a brace:

EI=E⋅I

Where I is the second moment of area of the brace cross-section. For a rectangular brace of width b and height/depth d:

I=b⋅d312

So EI = E × (b × d³) / 12, and since stiffness scales with d³, the depth (height) of a brace dominates its contribution enormously.

Eq. 4.4-4 — Measuring brace Young's modulus by a three-point bend or tap tone test on the brace stock itself.

Brace design criteria (section 4.4.2): Gore targets a specific total soundboard flexural rigidity sufficient to keep the bridge rotation under ~2° under string tension (roughly 10 Nm of torque from the strings). The deflection formula used is:

δ=F⋅L348⋅EI

The target is to design your bracing so the combined EI of plate + braces meets the stiffness needed to limit bridge rotation, while keeping mass as low as possible for acoustic efficiency.

## Practical Summary

The whole workflow is:

- Tap your rectangular plate → get f_L, f_C, f_LC

- Measure plate geometry + mass → get ρ

- Plug into 4.5-2/3/4 → get E_L, E_C, G_LC

- Plug into 4.5-7 with your guitar body dimensions and chosen f parameter → get target plate thickness

- Plane/sand to that thickness

- Design braces so E×(b×d³/12) gives enough total soundboard stiffness to pass the 2° bridge rotation criterion

The actual equations are proprietary to the book, so the exact numerical constants (like 0.95977 in 4.5-7) come from Gore's derivation of plate vibration theory — you'll want the book to get them all precisely. It's worth noting that there was a known typo in first edition equation 4.5-1 (exponent should be 4, not 2 in the denominator).

How do you measure f_L, f_c, and f_LC using deflection instead of frequency response?

7:11 PM

This is an important distinction — Gore's method actually uses tap tone frequencies (dynamic method), not static deflection, to get those three modal frequencies. Static deflection measures something different.

Here's how to think about the two approaches:

## Tap Tone (Dynamic) Method — What Gore Uses for f_L, f_C, f_LC

You're finding the resonant frequencies of the free plate's vibration modes by tapping and analyzing the spectrum (FFT). The plate is supported at its nodal lines (held at ~22% of length/width depending on mode), tapped at the antinode, and a microphone + spectrum analyzer captures the peak frequency. This is exactly what GuitarTap would be doing.

- f_L: hold plate at ~22% from each end (crosswise nodal lines), tap center → longitudinal bending mode

- f_C: hold plate at ~22% from each long edge, tap center → cross-grain bending mode

- f_LC: hold at one corner, tap the diagonally opposite corner → torsional mode

These are frequency measurements, not deflection. You can't get f_L, f_C, or f_LC from static deflection.

## What Static Deflection Can Give You

Static deflection gives you Young's modulus directly via a three-point bend test — an alternative path to E_L or E_C without needing tap tones:

E=F⋅L348⋅δ⋅I

Where:

- F = applied force (N)

- L = span between supports (m)

- δ = deflection at center (m)

- I = b·d³/12 (second moment of area of the strip)

Gore mentions this in the book (around p. 1-25) and it correlates well with the dynamic tap tone method — typically within about 1%. You'd use it on a narrow strip cut from the plate, oriented along or across the grain depending on which modulus you want.

But you can't get G_LC (shear modulus / torsional stiffness) from a simple static deflection test — that still requires the torsional tap tone f_LC, or a more complex torsion test rig.

## Bottom Line

If you're building GuitarTap to implement Gore's method, the tap tone FFT approach is exactly right for all three modes. Static deflection is an alternative route to E_L and E_C only, and is more of a bench measurement than something easily automated. The dynamic method is faster and more practical for a luthier workflow.

How to calculate poison’s ratio for different types of wood?

7:20 PM

Good question — Poisson's ratio comes up in Gore's more advanced plate theory. Here's the practical picture:

## The Short Answer: You Usually Don't Calculate It

For wood, Poisson's ratio is generally not measured directly by luthiers — it's looked up from published material property tables or estimated from known relationships. Direct measurement requires specialized equipment (strain gauges measuring lateral strain simultaneously with longitudinal strain under load), which is impractical in a lutherie context.

## What Poisson's Ratio Means for Wood

Wood is orthotropic, so it actually has three independent Poisson's ratios depending on which plane you're loading:

- ν_LR — longitudinal load, radial deformation (most relevant for guitar plates)

- ν_LT — longitudinal load, tangential deformation

- ν_RL, ν_TL, ν_RT, ν_TR — the other combinations

For quartersawn guitar tops and backs, ν_LR is the most relevant, representing how the wood contracts across the grain when stressed along the grain.

## Typical Values for Common Guitar Woods

| Wood | ν_LR (approx.) |
| --- | --- |
| Sitka Spruce | 0.37 |
| Engelmann Spruce | 0.42 |
| Western Red Cedar | 0.38 |
| Indian Rosewood | 0.30 |
| Mahogany | 0.30 |
| Maple | 0.40 |

These values are fairly consistent within species and don't vary nearly as much as E_L or density do between individual pieces. This is why Gore and most luthier-oriented engineering approaches treat Poisson's ratio as a fixed lookup value rather than something measured per-piece.

## How It Relates to Gore's Equations

Gore's plate thickness equation (4.5-7) doesn't explicitly require you to input Poisson's ratio — it's effectively absorbed into the plate vibration theory constants (like that 0.95977 coefficient). The moduli E_L, E_C, and G_LC that you measure via tap tones already capture the combined stiffness behavior of the plate including Poisson effects. So for implementing the Gore thicknessing method, you don't need ν at all.

Poisson's ratio becomes more important if you're doing full finite element modeling of a guitar top, or working through the more advanced coupled plate equations in Gore's book where he derives things from first principles.

## If You Actually Need to Calculate It

The relationship between elastic constants for an orthotropic material includes the symmetry condition:

νLREL=νRLER

So if you know E_L and E_R (radial modulus, roughly equivalent to E_C for quartersawn wood) and can find one Poisson's ratio from the literature for that species, you can estimate the reciprocal. But in practice for lutherie, published tables from wood science literature (USDA Wood Handbook is a good source) give you everything you need.

How sensitive is the thickness value to the poisson ratio?

7:43 PM

Great question for GuitarTap — and the practical answer is reassuring:

## Poisson's Ratio Has Very Little Effect on Calculated Thickness

This is actually something Gore addresses directly. The plate thickness result from 4.5-7 is dominated by E_L and density, with E_C and G_LC having secondary influence, and Poisson's ratio having essentially negligible effect.

Here's why:

## Where Poisson's Ratio Enters the Equation

In full orthotropic plate theory, the effective plate stiffness involves a term like:

D=E⋅h312(1-νLR⋅νRL)

The denominator contains the Poisson product ν_LR × ν_RL. For wood, these values are roughly:

- ν_LR ≈ 0.37 (spruce)

- ν_RL ≈ 0.04 (much smaller, because radial stiffness is much lower)

So the product is approximately 0.37 × 0.04 ≈ 0.015

That means the denominator is (1 − 0.015) = 0.985 — essentially 1.0. A ±50% error in your Poisson's ratio estimate changes this product by about ±0.008, which changes the denominator by less than 1%, and since thickness scales as the cube root or similar power of stiffness, the effect on h is tiny — well under 0.1 mm.

## Compared to Sensitivity on Other Parameters

To put it in perspective, thickness from 4.5-7 is most sensitive to:

- E_L — dominant effect, thickness scales roughly as E_L^(−1/2) to E_L^(−1/3) depending on the form. A 10% error in E_L → ~3-5% error in thickness → significant (could be 0.1-0.2 mm)

- Density ρ — also significant, similar order to E_L

- E_C — secondary, modified by the (a/b)⁴ term which amplifies its importance for wider guitars

- G_LC — tertiary, the torsional mode contribution is relatively small

- Poisson's ratio — negligible, well below measurement uncertainty of everything else

## Practical Implication for GuitarTap

You can safely hardcode species-typical Poisson's ratio values (or simply use a fixed value like 0.37 for softwood tops, 0.30 for hardwood backs) without meaningfully affecting the accuracy of your thickness calculations. The effort of measuring it per-piece would be entirely wasted — your tap tone frequency accuracy matters far more.

The uncertainty in reading f_L or f_C from an FFT peak (even just ±1 Hz) will dominate your error budget long before Poisson's ratio becomes relevant.

Is there away to save complete chat to a file?

7:47 PM

There isn't a built-in "export chat" button in claude.ai right now, but there are a few practical workarounds:

Browser-based options:

- Print to PDF — use your browser's Print function (Cmd+P on Mac, Ctrl+P on Windows) and choose "Save as PDF". Captures the whole conversation visually.

- Select all + copy — select all text on the page (Cmd+A), copy, paste into a text editor or Word doc. Loses formatting but preserves content.

Browser extensions:

- Some Markdown or webpage clipper extensions (like MarkDownload or SingleFile) can save the page content in a cleaner format.

For your use case specifically — since this conversation has a lot of mathematical content and equations, the Print to PDF route will preserve the rendered LaTeX and tables best. A raw text copy will lose the equation formatting.

It's worth submitting feedback to Anthropic requesting a native export feature — the thumbs down button on any response has a feedback option, or there's a feedback link in the interface. It's a commonly requested feature.

Claude is AI and can make mistakes. Please double-check responses.